### Chapter 2: Memory Technology

2.1 Hierarchical Memory Structures & Locality

- What is memory hierarchy? How do the different hierarchies of memory help to enhance the performance of the computer? Explain. (2025 Fall)

## What is Memory Hierarchy? How do the different hierarchies of memory help to enhance the performance of the computer? Explain.

### What is a Memory Hierarchy?

A memory hierarchy is an organized structure of different memory types arranged in several levels to balance performance, capacity, and cost.

As we move further from the processor:

- Memory capacity increases.

- Access time increases (speed decreases).

- Cost per byte decreases.

- Transfer bandwidth decreases.

- Unit of transfer size increases.

### Levels of Memory Hierarchy

- Register Memory: Fastest, smallest, and located inside the CPU. It holds operands and instructions currently being processed.

- Cache Memory (L1, L2, L3): Small, high-speed SRAM that stores frequently accessed data and instructions.

- Main Memory / RAM: Usually DRAM, which stores active programs and data during execution.

- Secondary Storage: SSDs and magnetic hard disks provide large, non-volatile storage but are slower than RAM.

- Backup Storage: Magnetic tapes or optical disks are mainly used for off-line archival storage.

### How Memory Hierarchy Enhances Computer Performance

- Cushioning the Memory Wall:
Caches reduce the speed gap between the fast processor and slower main memory, reducing processor stall time.

- Exploiting Principle of Locality:
Memory hierarchy keeps frequently used data in faster levels using:

  - Temporal Locality: Recently accessed data is likely to be accessed again.

  - Spatial Locality: Nearby memory locations are likely to be accessed.

  - Sequential Locality: Program execution generally follows sequential order.

- Inclusion Property:
Data in an upper level is maintained as a subset of the next lower level. A cache miss retrieves the required block from the lower level.

- Virtual Memory and Demand Paging:
Virtual Memory uses secondary storage to logically expand RAM. Demand paging loads only active pages into physical RAM, supporting efficient multitasking.

- Translation Lookaside Buffer (TLB):
A TLB is a specialized cache that stores recently used virtual-to-physical address translations, reducing the overhead of address translation.

### Conclusion

Thus, memory hierarchy provides high speed at low cost by combining small, fast memory with large, slower memory. It improves overall computer performance by reducing access time, memory stalls, and effective memory cost.

- Locality of reference (Short Note, 2025 Fall Mock)

- What new memory technologies can be used to improve hierarchical memory systems and make computational tasks faster and more efficient? Explain. (CA / Old)

2.2 Memory Capacity Planning

- What are the factors to be considered during memory capacity planning? Explain. (2025 Fall Mock)

## What are the factors to be considered during memory capacity planning? Explain.

Memory capacity planning is the process of forecasting, analyzing, and allocating adequate memory resources to ensure optimal performance, reliability, and scalability of a system or application.

### 1. Workload Requirements

- Application Type: Different workloads such as databases, web servers, and AI/ML models require different memory.

- Concurrency Levels: The number of simultaneous processes, threads, or users directly affects memory requirements.

- Usage Patterns: Memory should be planned according to average and peak usage to handle workload spikes.

### 2. System Architecture

- Hardware Configuration: CPU-to-memory bandwidth, memory hierarchy latency, and network speeds affect memory utilization.

- Software Optimization: Inefficient code, unoptimized data structures, and memory leaks can increase memory requirements.

- Operating System: Virtual memory, demand paging, and swapping affect physical RAM utilization.

### 3. Future Scalability

- Growth Projections: Planning should consider future increases in users, transactions, and data volume.

- New Features: New software functionalities may require additional memory.

- Elasticity: The system should dynamically adapt and scale to changing workload variations.

### 4. Cost-Performance Trade-offs

Memory capacity should balance access speed, capacity, and cost. Fast memory such as SRAM cache is expensive, while slower memory such as DRAM and secondary storage is cheaper. Therefore, memory hierarchy should be optimized according to performance and budget.

### 5. Performance and Resource Analysis

Workload profiling, predictive modeling, resource pooling, benchmarking, and stress testing help identify memory requirements, bottlenecks, and future capacity needs.

### Conclusion

Therefore, memory capacity planning considers workload requirements, system architecture, future scalability, cost-performance trade-offs, and performance analysis to ensure efficient and reliable memory utilization.

- Memory capacity planning (Short Note, 2025 Fall New)

2.3 Cache Memories and Management

- What is cache memory? Explain the different cache replacement algorithms (2025 Fall)

Cache memory is a small, extremely high-speed, SRAM-based storage layer situated close to or directly on the CPU. It acts as a buffer between the CPU and slower main memory (DRAM).

Purpose: It minimizes data access latency, reduces memory bottlenecks, and improves CPU processing efficiency.

- Cache Hit: Requested data is found in the cache and delivered quickly.

- Cache Miss: Data is not found, so a block is fetched from main memory and loaded into the cache.

- Principle of Locality: Cache works mainly through:

  - Temporal Locality: Recently accessed data is likely to be accessed again.

  - Spatial Locality: Nearby memory locations are likely to be accessed soon.

### Cache Replacement Algorithms

When the cache is full and a new block needs to be loaded, a replacement algorithm decides which block to evict.

### 1. First-In, First-Out (FIFO)

FIFO uses a first-in, first-out queue. The block that entered the cache first is evicted first, regardless of how recently it was accessed.

Example:
If the cache contains A, B, C and D arrives, FIFO removes A because it is the oldest block.

Effectiveness: Simple and cheap to implement, but may evict frequently used blocks. It can also suffer from Belady's Anomaly.

### 2. Least Recently Used (LRU)

LRU relies on the Principle of Locality. It removes the block that has not been accessed for the longest period of time.

Example:
If the access order is A, B, C, A and the cache is full, B is the least recently used and may be evicted.

Effectiveness: Generally provides a high hit ratio because it preserves recently used data. However, tracking access order has higher hardware complexity.

### 3. Least Frequently Used (LFU)

LFU removes the block with the lowest cumulative access count. It maintains a frequency counter for each block.

Example:
If the cache contains A(5), B(2), C(1) and a new block arrives, LFU removes C because it has the lowest frequency.

Effectiveness: Works well for repetitive access patterns but may suffer from frequency accumulation/cache pollution, where an old block with a high count remains in the cache even when it is no longer used.

#### 4. Optimal (OPT / MIN)

- Replaces the block that will not be used for the longest period in the future.

- Provides the lowest possible miss rate.

- Impossible to implement in real hardware because future references are unknown.

- Used mainly as an analytical benchmark.

#### 5. Random Replacement

- Randomly selects a block for eviction.

- Simple and has low state-tracking overhead.

- Generally provides lower hit rates because it ignores program locality.

#### 6. Circular FIFO (Second-Chance / Clock)

- Uses a circular queue with a reference bit.

- A block with reference bit 1 gets a second chance and the bit is reset.

- The first block with reference bit 0 is evicted.

- Approximates LRU with lower complexity.

#### 7. Optimum Cache Partition (OCP)

- A compiler-assisted cache replacement policy used for EPIC architectures.

- Compiler analyzes memory references and logically partitions the cache.

- Uses cache hints to control cache staying time and reduce cache pollution.

### Conclusion

Cache replacement algorithms determine which block should be removed when the cache is full. LRU provides good performance, FIFO is simple, LFU considers frequency, OPT gives the theoretical best result, while Random, Second-Chance, and OCP provide alternative approaches for different system requirements.

- Direct vs. associative mapping (Differentiate, 2025 Fall)

## Direct Mapping vs. Associative Mapping vs. Set-Associative Mapping

| Feature | Direct Mapping | Associative Mapping | Set-Associative Mapping |
| --- | --- | --- | --- |
| Block Placement | Exactly one specific cache block frame. | Any available block frame. | Any block frame within a specific set. |
| Address Fields | Tag, Index, Word Offset. | Tag, Word Offset. | Tag, Set Index, Word Offset. |
| Search | One tag comparison. | Compares tag with all block tags in parallel. | Compares tag with (k) tags within the set. |
| Replacement Choice | No choice; incoming block replaces the existing block. | Full choice; any block can be evicted. | Restricted choice; block is selected within the set. |
| Hardware Complexity | Low | High | Moderate |
| Conflict Misses | High | Lowest | Moderate |
| Performance | Fast but may have more conflict misses. | High hit ratio but expensive. | Good balance of cost and performance. |

### 1. Direct Mapping

- Every main memory block maps to exactly one specific cache block frame.

- Uses Tag, Index, and Word Offset.

- It is simple, fast, and has low hardware cost.

- It can suffer from conflict misses when multiple blocks map to the same cache location.

- No replacement policy is required.

### 2. Associative Mapping (Fully Associative)

- Any main memory block can be placed in any cache block frame.

- Uses Tag and Word Offset.

- Searches all cache tags in parallel using associative search/CAM.

- Provides maximum flexibility and the lowest conflict misses.

- Hardware is complex and expensive.

### 3. Set-Associative Mapping

- Cache block frames are divided into sets.

- A memory block maps to one specific set but can occupy any frame within that set.

- Uses Tag, Set Index, and Word Offset.

- Provides a balance between direct and associative mapping.

- Has moderate hardware complexity and reduces conflict misses compared with direct mapping.

### Conclusion

Direct mapping is simplest and cheapest, associative mapping provides maximum flexibility and hit ratio, while set-associative mapping provides a balance between performance, cost, and complexity.

- How do FIFO, LRU, and LFU cache replacement policies differ in terms of their approaches and effectiveness? Discuss with examples. (2024 Fall)

Cache replacement policies determine which cache block is removed when the cache is full and a new block needs to be loaded.

| Feature | FIFO | LRU | LFU |
| --- | --- | --- | --- |
| Approach | Evicts the oldest block. | Evicts the block not accessed for the longest time. | Evicts the block with the lowest access count. |
| Logic | Arrival order. | Temporal locality. | Access frequency. |
| Complexity | Low | High | High |
| Effectiveness | Suboptimal; ignores locality. | Excellent; generally gives a high hit ratio. | Moderate; effective for repetitive workloads. |
| Limitation | Can suffer from Belady's Anomaly. | High overhead for tracking access order. | Cache pollution due to frequency accumulation. |

### 1. First-In, First-Out (FIFO)

FIFO uses a first-in, first-out queue. The block that entered the cache first is evicted first, regardless of how recently it was accessed.

Example:
If the cache contains A, B, C and D arrives, FIFO removes A because it is the oldest block.

Effectiveness: Simple and cheap to implement, but may evict frequently used blocks. It can also suffer from Belady's Anomaly.

### 2. Least Recently Used (LRU)

LRU relies on the Principle of Locality. It removes the block that has not been accessed for the longest period of time.

Example:
If the access order is A, B, C, A and the cache is full, B is the least recently used and may be evicted.

Effectiveness: Generally provides a high hit ratio because it preserves recently used data. However, tracking access order has higher hardware complexity.

### 3. Least Frequently Used (LFU)

LFU removes the block with the lowest cumulative access count. It maintains a frequency counter for each block.

Example:
If the cache contains A(5), B(2), C(1) and a new block arrives, LFU removes C because it has the lowest frequency.

Effectiveness: Works well for repetitive access patterns but may suffer from frequency accumulation/cache pollution, where an old block with a high count remains in the cache even when it is no longer used.

### Conclusion

FIFO is simple but less effective, LRU is generally more effective because it exploits temporal locality, while LFU is useful for frequently accessed data but can suffer from cache pollution.

- Describe three cache replacement policies with appropriate examples. (2025 Fall New)

2.4 Virtual Memory Technology & Page Replacement

- Belady's Anomaly (Short Note, 2025 Fall)

## Belady's Anomaly

Belady's Anomaly is a phenomenon in virtual memory where increasing the number of physical page frames results in an increase in the number of page faults for a particular page reference sequence.

### Why Does It Occur?

- It mainly occurs with the FIFO (First-In, First-Out) page replacement algorithm.

- FIFO replaces pages based on their arrival order, without considering locality or frequency of use.

- Adding more frames can change the replacement order and cause additional page faults.

### Example

For the reference string:

1, 2, 3, 4, 1, 2, 5, 1, 2, 3, 4, 5

| Number of Frames | Page Faults |
| --- | --- |
| 3 Frames | 9 |
| 4 Frames | 10 |

Thus, increasing memory from 3 to 4 frames increases page faults from 9 to 10.

### Important Point

- FIFO → Can suffer from Belady's Anomaly.

- LRU and OPT → Do not suffer from Belady's Anomaly because they are stack algorithms with the inclusion property.

In short: Belady's Anomaly means more memory can sometimes cause more page faults, mainly in FIFO.

- Virtual memory (Short Note, 2024 Fall)

- What is page replacement and why is it essential? Compare the optimal page replacement algorithm with the FIFO algorithm using suitable examples. (2024 Fall)

## What is Page Replacement and Why is it Essential? Compare the Optimal Page Replacement Algorithm with the FIFO Algorithm using suitable examples.

### What is Page Replacement?

Page replacement is the process of removing a page from physical RAM and replacing it with a newly requested page when all available page frames are occupied.

### Why is Page Replacement Essential?

- Overcomes Physical RAM Limits: Allows programs to use a virtual address space larger than physical RAM.

- Enables Multiprogramming: Allows more processes to run by loading only required pages using demand paging.

- Handles Page Faults: When a required page is not in RAM, a page fault occurs and an existing page must be replaced.

- Improves Performance: An efficient replacement policy minimizes expensive page faults and improves memory utilization.

## Comparison of FIFO and Optimal Page Replacement

| Feature | FIFO | Optimal (OPT) |
| --- | --- | --- |
| Approach | Replaces the page that has been in memory the longest. | Replaces the page that will not be used for the longest time in the future. |
| Basis | Arrival order. | Future memory references. |
| Implementation | Simple queue. | Cannot be practically implemented. |
| Performance | May produce more page faults. | Produces the minimum possible page faults. |
| Limitation | Can suffer from Belady's Anomaly. | Requires knowledge of future references. |
| Use | Practical page replacement method. | Theoretical benchmark. |

### Example

Consider the page reference string:

[
0,1,2,4,2,3,7,2,1,3,1
]

with 3 page frames.

### 1. FIFO

FIFO removes the oldest page when a page fault occurs.

- Total Page Faults = 9

- Total Hits = 2

- Hit Ratio = 2/11 ≈ 18.1%

Example: When page 4 is requested, page 0 is removed because it entered memory first.

### 2. Optimal (OPT)

OPT removes the page that will be used farthest in the future.

- Total Page Faults = 7

- Total Hits = 4

- Hit Ratio = 4/11 ≈ 36.3%

Example: When page 4 is requested, page 0 is removed because it is never used again.

### Conclusion

FIFO is simple and practical but may cause more page faults and suffer from Belady's Anomaly. Optimal provides the minimum possible page faults but cannot be implemented in practice because future memory references are unknown. Therefore, OPT is mainly used as a benchmark for evaluating practical algorithms.

- What is a page fault? When does it occur? How is it handled? Explain. (2025 Fall Mock & 2025 Fall New)

### What is a Page Fault?

A page fault is a type of virtual memory miss that occurs when a running program accesses a memory page that is mapped in its virtual address space but is not currently present in physical RAM.

The operating system uses demand paging, where pages are loaded into RAM only when they are required.

### When Does a Page Fault Occur?

A page fault occurs when:

- Instruction or Data Referencing: A program accesses an instruction or data located on a page not currently in RAM.

- Present/Valid Bit Failure: The MMU or TLB detects that the page table entry has an invalid present/valid bit.

- Access Rights Violation: A memory access violates protection rules, such as writing to a read-only page.

### How is a Page Fault Handled?

- Pipeline Interruption: The processor stops the faulting instruction and saves the Program Counter (PC).

- Process Suspension: The OS suspends the faulting process and may perform a context switch to another process.

- Locate the Page: The OS locates the missing page on secondary storage.

- Page Replacement: If RAM is full, the OS selects a page using a replacement policy such as LRU, FIFO, or LFU. A modified page is written back to disk.

- Load the Page: The required page is loaded into a physical memory frame.

- Update Tables: The OS updates the page table and invalidates stale TLB entries.

- Resume Execution: The process is returned to the ready queue, and the faulting instruction is restarted using the saved PC.

### Conclusion

A page fault occurs when a required page is not present in RAM. The operating system handles it by locating the page, performing page replacement if necessary, updating the page table and TLB, and restarting the interrupted instruction.

- Which page replacement algorithm works best for improving memory usage and system performance? Explain. (CA / Old)

Least Recently Used (LRU) is generally considered the best practical page replacement algorithm because it follows the Principle of Locality and reduces page faults.

### 1. Optimal (OPT) Algorithm

- Replaces the page that will not be used for the longest period in the future.

- It gives the lowest page fault rate and highest hit ratio.

- However, it is not practically implementable because future page references are unknown.

- Therefore, it is mainly used as a theoretical benchmark.

### 2. Least Recently Used (LRU) Algorithm

- Replaces the page that has not been accessed for the longest time.

- It follows temporal locality: recently used pages are likely to be used again.

- It keeps the active working set in physical memory and reduces expensive page faults.

- Its main limitation is higher overhead for tracking page access history.

- In practice, Clock/Second-Chance and Pseudo-LRU are often used as approximations.

### 3. Comparison with Other Algorithms

- FIFO: Simple, but may remove frequently used pages and can suffer from Belady's Anomaly.

- LFU: Uses access frequency but may suffer from frequency accumulation and cache pollution.

- LRU: Adapts better to program locality and generally provides better performance.

### Conclusion

OPT is best theoretically, while LRU is the best practical choice for improving memory usage and system performance because it reduces page faults by exploiting the Principle of Locality.

- What is translation lookaside buffer (TLB)? Explain its roles in virtual memory systems and how it improves performance. (2025 Fall New)

## What is Translation Lookaside Buffer (TLB)? Explain its roles in virtual memory systems and how it improves performance.

### What is TLB?

A Translation Lookaside Buffer (TLB) is a fast hardware cache used to store recently used virtual-to-physical address translations. It is integrated into the Memory Management Unit (MMU) and helps reduce the time required for address translation.

A TLB entry mainly contains a virtual page number and physical page frame number, along with status bits such as valid, dirty, and protection bits.

### Roles of TLB in Virtual Memory

- Stores Address Translations:
Stores recently used virtual page-to-physical page frame mappings.

- TLB Lookup:
The virtual page number is used to search the TLB.

- TLB Hit:
If the translation is found, the physical page frame number is obtained quickly and the physical address is generated without accessing the page table.

- TLB Miss:
If the translation is not found, the processor accesses the page table in main memory. The obtained translation is then loaded into the TLB.

- Reduces Two-Memory-Access Problem:
Without a TLB, virtual memory may require one memory access for the page table and another for the actual data. TLB reduces this overhead.

### How TLB Improves Performance

- Exploits Locality: Frequently used page translations remain in the TLB due to temporal and spatial locality.

- Reduces Effective Access Time (EAT): A high TLB hit ratio significantly reduces memory access time.

- Removes Translation Delay: TLB allows address translation to occur quickly and can operate in parallel with cache lookup.

- Improves Context Switching: ASIDs (Address Space Identifiers) allow translations from different processes to remain in the TLB, reducing the need for complete TLB flushing.

### Conclusion

The TLB is an important part of the virtual memory system. By caching recently used address translations, it reduces page-table accesses, lowers effective access time, and improves overall system performance.